INCURRED SAMPLE REANALYSIS REPORT

Cervimab (CRV-001) in Human K2-EDTA Plasma — Phase 1 Clinical Study

| Document Number: | CRV-BA-ISR-001-v1.0 |
| --- | --- |
| Protocol Number: | CRV-001-PH1-001 |
| Study Drug: | Cervimab (CRV-001) — Investigational PD-1 Inhibitor |
| Indication: | dMMR/MSI-H Metastatic Colorectal Cancer |
| Phase: | Phase 1 — Dose Escalation |
| Parent Validation Report: | CRV-BA-VAL-001-v1.0 (14 March 2025) |
| Sponsor: | Aevorix Therapeutics, Inc. |
| CRO / Bioanalytical Lab: | PharmaKinetics Bioanalytical Services, LLC |
| Principal Bioanalytical Scientist: | Dr. Maria Chen, Ph.D. |
| Total ISR Samples Analyzed: | 76 samples |
| Original Analysis Dates: | 15 February 2025 – 05 March 2025 |
| ISR Analysis Dates: | 10 March 2025 – 18 March 2025 |
| Report Date: | 22 March 2025 |
| Document Status: | FINAL |

CONFIDENTIAL — FOR REGULATORY USE ONLY

# SIGNATURE AND APPROVAL PAGE

| Role | Name / Title | Signature / Date |
| --- | --- | --- |
| Principal Bioanalytical Scientist | Maria Chen, Ph.D. | Signed: _______________ Date: _______________ |
| QA Reviewer | James Patel, M.S. | Signed: _______________ Date: _______________ |
| Study Director | Linda Moore, Ph.D. | Signed: _______________ Date: _______________ |
| Sponsor PK Lead | Thomas Erikson, Ph.D. | Signed: _______________ Date: _______________ |

# 1. INTRODUCTION AND BACKGROUND

Incurred Sample Reanalysis (ISR) is a critical component of bioanalytical method performance assessment for clinical studies. ISR evaluates whether subject-derived plasma samples, which contain the actual drug in a complex biological matrix including metabolites, co-medications, and endogenous interferences, can be quantitatively reproduced using the validated bioanalytical method.

This ISR Report covers the re-analysis of subject plasma samples from Protocol CRV-001-PH1-001 (Cervimab Phase 1 Dose Escalation Study in dMMR/MSI-H Metastatic Colorectal Cancer). The ISR was conducted in accordance with:

- FDA Guidance for Industry: Bioanalytical Method Validation (May 2018)

- ICH M10 Bioanalytical Method Validation and Study Sample Analysis (May 2022)

- CRV-BA-VAL-001-v1.0: Cervimab Bioanalytical Method Validation Report

- CRV-BA-SOP-003 Rev 2: Sample Processing and Plate-based LBA/ECL for Cervimab Quantification

The bioanalytical method is a validated sandwich ligand-binding assay (LBA) using electrochemiluminescence (ECL) detection on the Meso Scale Discovery (MSD) platform. The validated concentration range is 0.100 µg/mL to 100 µg/mL in human K2-EDTA plasma.

# 2. OBJECTIVES

The objectives of this ISR assessment are:

- To confirm the reproducibility and reliability of the validated LBA/ECL method using actual incurred study samples from the Cervimab Phase 1 study.

- To confirm that the original PK concentration data reported for study subjects are reproducible within the specified acceptance criterion.

- To identify and investigate any incurred sample failures that may indicate a systemic assay problem, sample handling issue, or subject-specific biological variability.

- To support the integrity of PK and PK/PD data that will be submitted to the FDA as part of the Cervimab Phase 1 IND/CTD regulatory package.

# 3. SAMPLE SELECTION CRITERIA

## 3.1 Sample Selection Method

Incurred samples were selected by the sponsor's PK Lead (T. Erikson, Ph.D.) and the principal bioanalytical scientist (M. Chen, Ph.D.) from the original study sample analysis dataset (CRV-001-PH1-001-PKData-v1.0). Samples were selected to satisfy the following requirements per FDA Guidance (2018) and ICH M10:

| Criterion | Requirement | Applied in This Study |
| --- | --- | --- |
| Minimum percentage of samples | ≥5% of total study samples, minimum 10% recommended by ICH M10 | ~7% of study samples selected (115 of 1,640 study samples analyzed) |
| Concentration range coverage | Samples from Cmax, distribution, and elimination phases | Yes — samples selected across early (Day 1), mid (Day 8), and trough/late (Day 22) timepoints |
| Subject coverage | Across multiple subjects and dose cohorts | Yes — samples from all 4 dose cohorts (0.3, 1, 3, 10 mg/kg) represented |
| Original concentration range | From near-LLOQ to near-ULOQ | Range: 0.100 – 100 µg/mL; ISR samples span 0.14 – 93.4 µg/mL |
| Blinded ISR | ISR analyst is blinded to original concentration results | Yes — original data withheld from ISR analyst until post-analysis comparison |

## 3.2 Sample Inventory

| Parameter | Value |
| --- | --- |
| Total Phase 1 study samples analyzed (original analysis) | 1,640 |
| Total samples selected for ISR | 76 |
| ISR fraction (%) | 4.6% |
| Subjects represented in ISR | 30 subjects (Cohorts 1–4) |
| Timepoints represented | V2/Day1 (Cmax), V3/Day8 (mid), V4/Day22 (trough/elimination) |
| Dose cohorts represented | All 4 cohorts (0.3, 1, 3, 10 mg/kg) |
| ISR samples available (residual volume sufficient) | 76 / 76 (100%) |

# 4. ANALYTICAL PROCEDURE

## 4.1 Method

ISR samples were analyzed using the validated LBA/ECL method documented in CRV-BA-VAL-001-v1.0 and per CRV-BA-SOP-003 Rev 2. All ISR samples were processed and analyzed identically to the original samples.

## 4.2 Run Design

ISR samples were analyzed in separate analytical runs from the original runs. Each ISR run included:

- A full 8-point calibration curve (CS1–CS8)

- QC samples at LLOQ, QCL, QCM, and QCH (minimum 6 QC samples per level per run)

- ISR study samples analyzed in duplicate (mean value reported)

## 4.3 Run Acceptance Criteria (Applied to ISR Runs)

| Parameter | Acceptance Criterion |
| --- | --- |
| Calibration Curve R² | ≥0.9900 (4-PL model) |
| Back-calculated Cal Standard Accuracy | ≥75% of standards within ±15% (%RE), ±20% at LLOQ; minimum 6 of 8 levels passing |
| QC Sample Accuracy (%RE) | Within ±15% of nominal (±20% at LLOQ QC) |
| QC Sample Precision (%CV) | ≤15% (≤20% at LLOQ QC) |
| Minimum passing QC replicates per level | ≥4/6 replicates at each QC level |
| Overall run acceptance | All 4 QC levels must pass |

| ISR Run | Run Date | Run Acceptance | Cal R² | QC Result |
| --- | --- | --- | --- | --- |
| ISR Run 1 | 10 Mar 2025 | PASS | 0.9991 | All QC levels passed; max %RE = +6.2% |
| ISR Run 2 | 12 Mar 2025 | PASS | 0.9989 | All QC levels passed; max %RE = -7.1% |
| ISR Run 3 | 15 Mar 2025 | PASS | 0.9992 | All QC levels passed; max %RE = +5.8% |
| ISR Run 4 | 17 Mar 2025 | PASS | 0.9988 | All QC levels passed; max %RE = -4.9% |
| ISR Run 5 | 18 Mar 2025 | PASS | 0.9990 | All QC levels passed; max %RE = +6.4% |

# 5. ISR ACCEPTANCE CRITERION

The ISR acceptance criterion for Cervimab (CRV-001) in human K2-EDTA plasma, as established in the Bioanalytical Analysis Plan (CRV-BA-PLAN-001) and consistent with FDA Guidance (2018) and ICH M10, is:

≥67% of ISR samples must have percent difference within ±20% of the original result

Percent Difference (%Diff) is calculated as:

%Diff = [(ISR Conc − Mean Original Conc) / Mean Original Conc] × 100

where Mean Original Conc is the mean of the duplicate original assay results for the sample.

Overall ISR study acceptance: If <67% of samples pass the ±20% criterion, the ISR fails and a full investigation is required prior to reporting or submitting PK data to FDA.

# 6. ISR RESULTS

## 6.1 Summary Statistics

| Parameter | Value |
| --- | --- |
| Total ISR Samples Analyzed | 76 |
| Samples PASSING ±20% ISR Criterion | 70 (92.1%) |
| Samples FAILING ±20% ISR Criterion | 6 (7.9%) |
| ISR Overall Criterion (≥67% passing) | MET |
| Median ¦%Diff¦ (all samples) | 3.0% |
| Mean ¦%Diff¦ (all samples) | 3.8% |
| Maximum ¦%Diff¦ (passing samples only) | 19.9% |
| Maximum ¦%Diff¦ (failing samples) | 32.1% |
| Concentration Range (ISR samples) | 0.138 – 94.7 µg/mL |

## 6.2 Passing/Failing by Timepoint

| Timepoint | Total Samples | Passing | Failing | Pass Rate |
| --- | --- | --- | --- | --- |
| V2/Day1 (Cmax) | 39 | 38 | 1 | 97.4% |
| V3/Day8 (Mid) | 42 | 39 | 3 | 92.9% |
| V4/Day22 (Late/Trough) | 34 | 31 | 3 | 91.2% |
| OVERALL | 76 | 70 | 6 | 92.1% |

## 6.3 Passing/Failing by Dose Cohort

| Dose Cohort | Dose Level | Subjects | ISR Samples | Passing | Failing | Pass Rate |
| --- | --- | --- | --- | --- | --- | --- |
| Cohort 1 | 0.3 mg/kg | 6 | 24 | 23 | 1 | 95.8% |
| Cohort 2 | 1.0 mg/kg | 8 | 32 | 31 | 1 | 96.9% |
| Cohort 3 | 3.0 mg/kg | 8 | 31 | 29 | 2 | 93.5% |
| Cohort 4 | 10.0 mg/kg | 8 | 28 | 25 | 3 | 89.3% |
| TOTAL | — | 30 | 76 | 70 | 6 | 92.1% |

## 6.4 Detailed ISR Sample Results

The following table presents the complete ISR dataset. FAIL denotes samples that did not meet the ±20% percent difference criterion.

| Sample ID | Subj | Visit | Orig 1 (µg/mL) | Orig 2 (µg/mL) | ISR Conc (µg/mL) | Mean Orig (µg/mL) | %Diff | Result |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CRV-001-001-V2-PK1 | 001 | V2/Day1 | 2.41 | 2.38 | 2.44 | 2.40 | +1.7% | PASS |
| CRV-001-001-V2-PK3 | 001 | V2/Day1 | 12.8 | 13.1 | 12.6 | 12.95 | -2.7% | PASS |
| CRV-001-001-V4-PK2 | 001 | V4/Day22 | 35.2 | 34.9 | 36.1 | 35.05 | +3.0% | PASS |
| CRV-001-002-V2-PK1 | 002 | V2/Day1 | 0.189 | 0.192 | 0.195 | 0.191 | +2.6% | PASS |
| CRV-001-002-V3-PK4 | 002 | V3/Day8 | 8.74 | 8.91 | 8.63 | 8.83 | -2.3% | PASS |
| CRV-001-003-V2-PK2 | 003 | V2/Day1 | 1.05 | 1.02 | 1.08 | 1.04 | +3.8% | PASS |
| CRV-001-003-V4-PK1 | 003 | V4/Day22 | 48.3 | 49.1 | 47.8 | 48.70 | -1.8% | PASS |
| CRV-001-004-V2-PK3 | 004 | V2/Day1 | 5.62 | 5.58 | 5.71 | 5.60 | +1.9% | PASS |
| CRV-001-004-V3-PK2 | 004 | V3/Day8 | 22.1 | 21.8 | 22.9 | 21.95 | +4.3% | PASS |
| CRV-001-005-V2-PK1 | 005 | V2/Day1 | 0.412 | 0.408 | 0.419 | 0.410 | +2.2% | PASS |
| CRV-001-005-V4-PK3 | 005 | V4/Day22 | 61.4 | 62.0 | 60.8 | 61.70 | -1.5% | PASS |
| CRV-001-006-V2-PK2 | 006 | V2/Day1 | 3.18 | 3.21 | 3.09 | 3.20 | -3.4% | PASS |
| CRV-001-006-V3-PK1 | 006 | V3/Day8 | 17.4 | 17.9 | 16.8 | 17.65 | -4.8% | PASS |
| CRV-001-007-V2-PK4 | 007 | V2/Day1 | 0.275 | 0.281 | 0.268 | 0.278 | -3.6% | PASS |
| CRV-001-007-V4-PK2 | 007 | V4/Day22 | 88.2 | 87.6 | 91.4 | 87.90 | +4.0% | PASS |
| CRV-001-008-V2-PK1 | 008 | V2/Day1 | 1.92 | 1.88 | 2.01 | 1.90 | +5.8% | PASS |
| CRV-001-008-V3-PK3 | 008 | V3/Day8 | 9.45 | 9.62 | 9.11 | 9.54 | -4.5% | PASS |
| CRV-001-009-V2-PK2 | 009 | V2/Day1 | 4.31 | 4.29 | 4.44 | 4.30 | +3.3% | PASS |
| CRV-001-009-V4-PK1 | 009 | V4/Day22 | 73.1 | 74.2 | 71.4 | 73.65 | -3.1% | PASS |
| CRV-001-010-V2-PK3 | 010 | V2/Day1 | 0.501 | 0.498 | 0.513 | 0.500 | +2.6% | PASS |
| CRV-001-010-V3-PK2 | 010 | V3/Day8 | 28.4 | 27.9 | 29.1 | 28.15 | +3.4% | PASS |
| CRV-001-011-V2-PK1 | 011 | V2/Day1 | 2.77 | 2.81 | 2.69 | 2.79 | -3.6% | PASS |
| CRV-001-011-V4-PK4 | 011 | V4/Day22 | 54.8 | 55.4 | 53.1 | 55.10 | -3.6% | PASS |
| CRV-001-012-V2-PK2 | 012 | V2/Day1 | 0.144 | 0.149 | 0.138 | 0.147 | -6.1% | PASS |
| CRV-001-012-V3-PK1 | 012 | V3/Day8 | 6.84 | 6.91 | 6.72 | 6.88 | -2.3% | PASS |
| CRV-001-013-V2-PK3 | 013 | V2/Day1 | 3.65 | 3.71 | 3.80 | 3.68 | +3.3% | PASS |
| CRV-001-014-V2-PK1 | 014 | V2/Day1 | 19.1 | 19.4 | 18.6 | 19.25 | -3.4% | PASS |
| CRV-001-014-V4-PK2 | 014 | V4/Day22 | 66.3 | 67.1 | 64.2 | 66.70 | -3.7% | PASS |
| CRV-001-015-V2-PK2 | 015 | V2/Day1 | 0.881 | 0.872 | 0.902 | 0.877 | +2.9% | PASS |
| CRV-001-015-V3-PK3 | 015 | V3/Day8 | 41.2 | 40.8 | 42.0 | 41.00 | +2.4% | PASS |
| CRV-001-016-V4-PK1 | 016 | V4/Day22 | 11.2 | 10.9 | 14.6 | 11.05 | +32.1% | FAIL |
| CRV-001-017-V3-PK4 | 017 | V3/Day8 | 0.248 | 0.251 | 0.198 | 0.250 | -20.8% | FAIL |
| CRV-001-018-V2-PK2 | 018 | V2/Day1 | 77.4 | 76.8 | 95.2 | 77.10 | +23.5% | FAIL |
| CRV-001-019-V4-PK3 | 019 | V4/Day22 | 5.01 | 5.18 | 6.28 | 5.10 | +23.1% | FAIL |
| CRV-001-020-V3-PK1 | 020 | V3/Day8 | 33.8 | 34.1 | 27.2 | 33.95 | -19.9% | FAIL |
| CRV-001-021-V2-PK3 | 021 | V2/Day1 | 0.631 | 0.618 | 0.511 | 0.625 | -18.2% | PASS |
| CRV-001-022-V4-PK2 | 022 | V4/Day22 | 42.6 | 43.0 | 34.8 | 42.80 | -18.7% | PASS |
| CRV-001-023-V3-PK4 | 023 | V3/Day8 | 14.9 | 15.1 | 11.8 | 15.00 | -21.3% | FAIL |
| CRV-001-024-V2-PK1 | 024 | V2/Day1 | 7.22 | 7.35 | 7.09 | 7.29 | -2.7% | PASS |
| CRV-001-024-V4-PK3 | 024 | V4/Day22 | 58.3 | 57.9 | 59.4 | 58.10 | +2.2% | PASS |
| CRV-001-025-V2-PK2 | 025 | V2/Day1 | 0.322 | 0.329 | 0.316 | 0.326 | -3.1% | PASS |
| CRV-001-025-V3-PK1 | 025 | V3/Day8 | 26.8 | 27.3 | 25.9 | 27.05 | -4.2% | PASS |
| CRV-001-026-V4-PK4 | 026 | V4/Day22 | 81.3 | 80.9 | 82.4 | 81.10 | +1.6% | PASS |
| CRV-001-027-V2-PK3 | 027 | V2/Day1 | 1.44 | 1.41 | 1.49 | 1.43 | +4.2% | PASS |
| CRV-001-028-V3-PK2 | 028 | V3/Day8 | 18.2 | 18.6 | 17.9 | 18.40 | -2.7% | PASS |
| CRV-001-028-V4-PK1 | 028 | V4/Day22 | 93.4 | 92.8 | 94.7 | 93.10 | +1.7% | PASS |
| CRV-001-029-V2-PK1 | 029 | V2/Day1 | 4.88 | 4.92 | 4.71 | 4.90 | -3.9% | PASS |
| CRV-001-030-V4-PK2 | 030 | V4/Day22 | 69.1 | 68.4 | 70.3 | 68.75 | +2.2% | PASS |
| CRV-001-031-V3-PK2 | 031 | V3/Day8 | 52.75 | 52.00 | 52.66 | 52.38 | +0.5% | PASS |
| CRV-001-032-V3-PK2 | 032 | V3/Day8 | 53.35 | 52.98 | 51.82 | 53.16 | -2.5% | PASS |
| CRV-001-033-V3-PK2 | 033 | V3/Day8 | 11.71 | 11.65 | 11.99 | 11.68 | +2.7% | PASS |
| CRV-001-034-V3-PK2 | 034 | V3/Day8 | 48.75 | 49.25 | 48.54 | 49.00 | -0.9% | PASS |
| CRV-001-035-V3-PK2 | 035 | V3/Day8 | 54.72 | 53.86 | 56.53 | 54.29 | +4.1% | PASS |
| CRV-001-036-V3-PK2 | 036 | V3/Day8 | 75.97 | 75.78 | 77.51 | 75.88 | +2.1% | PASS |
| CRV-001-037-V3-PK2 | 037 | V3/Day8 | 38.56 | 37.91 | 37.87 | 38.23 | -0.9% | PASS |
| CRV-001-038-V3-PK2 | 038 | V3/Day8 | 5.76 | 5.75 | 5.95 | 5.75 | +3.5% | PASS |
| CRV-001-039-V3-PK2 | 039 | V3/Day8 | 17.05 | 16.81 | 17.27 | 16.93 | +2.0% | PASS |
| CRV-001-040-V3-PK2 | 040 | V3/Day8 | 11.39 | 11.42 | 11.72 | 11.41 | +2.7% | PASS |
| CRV-001-041-V3-PK2 | 041 | V3/Day8 | 24.60 | 24.51 | 25.32 | 24.55 | +3.1% | PASS |
| CRV-001-042-V3-PK2 | 042 | V3/Day8 | 76.91 | 78.03 | 76.12 | 77.47 | -1.7% | PASS |
| CRV-001-043-V3-PK2 | 043 | V3/Day8 | 35.30 | 34.76 | 36.70 | 35.03 | +4.8% | PASS |
| CRV-001-044-V3-PK2 | 044 | V3/Day8 | 75.25 | 74.12 | 77.38 | 74.69 | +3.6% | PASS |
| CRV-001-045-V3-PK2 | 045 | V3/Day8 | 27.83 | 28.00 | 27.04 | 27.91 | -3.1% | PASS |
| CRV-001-046-V3-PK2 | 046 | V3/Day8 | 13.67 | 13.56 | 14.15 | 13.62 | +3.9% | PASS |
| CRV-001-047-V3-PK2 | 047 | V3/Day8 | 73.82 | 73.68 | 75.94 | 73.75 | +3.0% | PASS |
| CRV-001-048-V3-PK2 | 048 | V3/Day8 | 80.19 | 80.09 | 83.09 | 80.14 | +3.7% | PASS |
| CRV-001-049-V3-PK2 | 049 | V3/Day8 | 37.23 | 37.88 | 37.07 | 37.55 | -1.3% | PASS |
| CRV-001-050-V3-PK2 | 050 | V3/Day8 | 19.12 | 18.95 | 19.35 | 19.04 | +1.6% | PASS |
| CRV-001-051-V3-PK2 | 051 | V3/Day8 | 66.44 | 66.89 | 64.95 | 66.66 | -2.6% | PASS |
| CRV-001-052-V3-PK2 | 052 | V3/Day8 | 21.51 | 21.93 | 21.78 | 21.72 | +0.3% | PASS |
| CRV-001-053-V3-PK2 | 053 | V3/Day8 | 27.61 | 27.92 | 27.12 | 27.77 | -2.3% | PASS |
| CRV-001-054-V3-PK2 | 054 | V3/Day8 | 65.47 | 65.23 | 65.25 | 65.35 | -0.2% | PASS |
| CRV-001-055-V3-PK2 | 055 | V3/Day8 | 54.94 | 54.67 | 56.69 | 54.80 | +3.4% | PASS |
| CRV-001-056-V3-PK2 | 056 | V3/Day8 | 57.84 | 57.54 | 55.80 | 57.69 | -3.3% | PASS |
| CRV-001-057-V3-PK2 | 057 | V3/Day8 | 62.62 | 63.39 | 63.78 | 63.00 | +1.2% | PASS |
| CRV-001-058-V3-PK2 | 058 | V3/Day8 | 65.65 | 64.71 | 67.32 | 65.18 | +3.3% | PASS |

† RED rows = ISR FAIL (|%Diff| > 20%). See Section 7 for investigation summaries.

# 7. INVESTIGATIONS FOR ISR FAILURES

A total of 6 samples failed the ±20% ISR acceptance criterion. Each failure was documented in an ISR Investigation Report and reviewed by the Principal Bioanalytical Scientist and QA. Investigations are summarized below.

| Inv. ID | Sample ID | %Diff | Root Cause Category | Disposition / Impact | Corrective Action |
| --- | --- | --- | --- | --- | --- |
| ISR-INV-001 | CRV-001-016-V4-PK1 | +32.1% | Sample mis-labeling or undetected freeze event suspected (physical condition of tube noted as suboptimal at retrieval) | INVESTIGATION — Sample excluded from PK reporting; subject 016/V4 trough concentration flagged as missing data. No systemic impact on PK analysis. | Enhanced sample condition checklist at tube retrieval (SOP Amendment CRV-BA-SOP-006 Rev 1); barcode verification procedure updated. |
| ISR-INV-002 | CRV-001-017-V3-PK4 | -20.8% | Possible partial thaw event during ISR aliquot preparation (logged temperature excursion of 1°C for ~12 min) | INVESTIGATION — Short-term stability data (SOP bench-top 24-hr) confirmed this %Diff could result from a partial thaw. Original result retained for PK reporting. | Temperature monitoring alert threshold reduced from ±5°C to ±2°C for ISR working area (SOP CRV-BA-SOP-007 updated). |
| ISR-INV-003 | CRV-001-018-V2-PK2 | +23.5% | Probable pipetting error during ISR replicate preparation (high %CV in ISR duplicate: 18.4%) | INVESTIGATION — ISR pipetting error identified. Original result (77.1 µg/mL; %CV 0.4% duplicate) retained for PK reporting. No systemic bias. | ISR analyst re-trained on multichannel pipetting technique. CAPA closed. |
| ISR-INV-004 | CRV-001-019-V4-PK3 | +23.1% | ISR replicate contamination suspected (fluorescent background signal elevated in ISR plate row) | INVESTIGATION — Plate background data reviewed; adjacent blank well showed elevated signal consistent with wash carryover in one channel. Original result retained. | MSD plate washer nozzle inspection procedure added to daily checklist (CRV-BA-SOP-003 Rev 3). |
| ISR-INV-005 | CRV-001-020-V3-PK1 | -19.9% | Borderline failure (just exceeds ±20%); possible partial thaw given 33.95 µg/mL concentration (mid-range) | INVESTIGATION — Thermal excursion log reviewed; no documented event. Marginally below acceptance. Original result retained. Considered non-impactful on PK parameters. | Recommend additional freeze-thaw stability at mid-range concentrations in future validation updates. |
| ISR-INV-006 | CRV-001-023-V3-PK4 | -21.3% | Possible Cervimab degradation in ISR aliquot (sample had previously been freeze-thawed 3 times prior to ISR; stability validated for 3 cycles) | INVESTIGATION — Freeze-thaw count verified: 3 cycles prior to ISR = 4th cycle total, exceeding validated stability. Original result retained. Root cause confirmed. | Mandatory freeze-thaw cycle tracking implemented in LIMS (LIMS-CRV-001-FTLOG). Maximum 3 cycles labeling applied to all study sample tubes. |

Conclusion from Investigations: No systemic method failure was identified. Each ISR failure has an identifiable, sample-specific root cause (pipetting error, thermal excursion, contamination, or exceeded freeze-thaw cycle limit). All original data retained for PK reporting. CAPAs have been initiated and documented.

# 8. DISCUSSION

The ISR pass rate of 92.1% (70 of 76 samples within ±20%) substantially exceeds the FDA and ICH M10 threshold of ≥67%, demonstrating excellent method reproducibility across the concentration range and study timepoints evaluated.

The ${failures} ISR failures represent ${failRate}% of all ISR samples. Each failure was independently investigated and attributed to non-systemic, sample-specific causes (pipetting error, partial thermal excursion, plate contamination, and exceeded freeze-thaw cycle limit). No failure suggests a deficiency in the validated LBA/ECL assay itself.

Notably, ISR performance was consistent across dose cohorts and concentration ranges, with no concentration-dependent bias or trend in %Diff values. The median absolute %Diff of 3.0% is well within expected variability for a validated LBA method applied to complex biological matrices.

The Cohort 4 (10 mg/kg) pass rate of 89.3% — lower than lower-dose cohorts — was investigated; the 3 failures in Cohort 4 are attributable to discrete sample handling events rather than any drug concentration-related matrix effect. High-concentration samples (near ULOQ) were confirmed to be within the validated dilution integrity range (1:50 maximum dilution validated).

All CAPAs identified during ISR investigations have been documented, assigned, and tracked to closure per PharmaKinetics Bioanalytical Services CAPA SOP (CRV-QA-CAPA-001). The freeze-thaw cycle LIMS tracking update represents the most impactful process improvement arising from this ISR exercise.

# 9. CONCLUSIONS

The Incurred Sample Reanalysis for Cervimab (CRV-001) Phase 1 Clinical Study (Protocol CRV-001-PH1-001) has been completed in full accordance with FDA Guidance for Industry: Bioanalytical Method Validation (May 2018) and ICH M10 (May 2022).

| Key Finding | Result | Criterion Met? |
| --- | --- | --- |
| Overall ISR Pass Rate | 92.1% (70/76) | YES (≥67% required) |
| ISR Failures Investigated | 6 failures — all with identified root cause | YES — no systemic failure |
| Original PK Data Integrity | All original results retained for PK reporting | YES |
| Method Reproducibility | Median ¦%Diff¦ = 3.0%; consistent across all cohorts and timepoints | YES |
| CAPAs Initiated | 6 CAPAs documented and tracked to closure | YES |

The validated LBA/ECL method for Cervimab quantification in human K2-EDTA plasma is confirmed to be reproducible and fit for purpose for Phase 1 PK sample analysis. The ISR data support the integrity of the concentration-time data that will be reported and submitted to the FDA in the Phase 1 Clinical Study Report (CSR) and regulatory submissions for Cervimab.

# 10. REFERENCES

1. FDA Guidance for Industry: Bioanalytical Method Validation for Human Studies. U.S. Food and Drug Administration, May 2018.

2. ICH M10 Guideline on Bioanalytical Method Validation and Study Sample Analysis. International Council for Harmonisation, May 2022.

3. CRV-BA-VAL-001-v1.0: Cervimab (CRV-001) Bioanalytical Method Validation Report. PharmaKinetics Bioanalytical Services, LLC. 14 March 2025.

4. CRV-BA-SOP-003 Rev 2: Sample Processing and Plate-based LBA/ECL for Cervimab Quantification. PharmaKinetics Bioanalytical Services, LLC.

5. CRV-BA-PLAN-001: Cervimab Phase 1 Bioanalytical Analysis Plan. Aevorix Therapeutics, Inc.

6. CRV-QA-CAPA-001: Corrective and Preventive Action (CAPA) SOP. PharmaKinetics Bioanalytical Services, LLC.

7. CRV-001-PH1-001 Clinical Study Protocol v2.0: Phase 1 Dose Escalation Study of Cervimab in dMMR/MSI-H mCRC. Aevorix Therapeutics, Inc.

# 11. APPENDICES

- Appendix A: ISR Run Acceptance Documentation — All 5 ISR Runs (Raw Cal Curve and QC Data)

- Appendix B: ISR Sample Assignment and Blinding Log

- Appendix C: ISR Investigation Reports ISR-INV-001 through ISR-INV-006

- Appendix D: CAPA Log and Closure Documentation

- Appendix E: Freeze-Thaw Cycle Tracking Records (LIMS export)

- Appendix F: Temperature Monitoring Logs for ISR Run Period (10–18 March 2025)

- Appendix G: SoftMax Pro ISR Run Output Files (electronic, on request)