DRUG PRODUCT BATCH MANUFACTURING RECORD

CERVIMAB (Anti-PD-1 Monoclonal Antibody)

200 mg/10 mL (20 mg/mL) Concentrate for Solution for Infusion

Sterile, Preservative-Free, Single-Use Vial

FOR EXAMPLE / ILLUSTRATIVE PURPOSES ONLY  –  NOT FOR MANUFACTURING USE

| Document Number: | CVM-CMC-BR-001 |
| --- | --- |
| IND Number: | Example IND 000001 |
| Product Name: | Cervimab (Anti-PD-1 mAb) |
| Batch Number: | CVM-DP-P1-001 |
| Batch Size: | 2.0 L (200 vials x 10.5 mL fill) |
| Manufacturing Site: | Example BioPharma GMP Facility, Suite 300, [City, State] |
| Manufacturing Date: | [DD-MMM-YYYY] |
| Clinical Phase: | Phase 1 (First-in-Human) |
| Indication: | dMMR/MSI-H Metastatic Colorectal Cancer |
| Prepared By: | _________________________ Date: ______ |
| Reviewed By: | _________________________ Date: ______ |
| Approved By (QA): | _________________________ Date: ______ |

# 1. PURPOSE AND SCOPE

This Batch Manufacturing Record (BMR) documents the manufacturing of Cervimab Drug Product (DP) Batch CVM-DP-P1-001, a sterile, preservative-free, concentrated solution of an anti-PD-1 monoclonal antibody intended for intravenous infusion. This batch is manufactured under current Good Manufacturing Practice (cGMP) conditions in compliance with 21 CFR Parts 210, 211, and 600–680 for use in Phase 1 clinical studies.

The drug product is manufactured from Drug Substance (DS) Batch CVM-DS-P1-001 (IgG4 anti-PD-1 monoclonal antibody, produced in CHO cells). The DP formulation consists of 20 mg/mL Cervimab in a histidine-buffered, sucrose-stabilized formulation with polysorbate 80.

# 2. REGULATORY AND DOCUMENT REFERENCES

| Reference | Document Title / Identifier |
| --- | --- |
| 21 CFR Part 211 | Current Good Manufacturing Practice for Finished Pharmaceuticals |
| 21 CFR Part 600-680 | Biologics Standards – General Provisions and Testing |
| ICH Q7 | Good Manufacturing Practice Guide for Active Pharmaceutical Ingredients |
| ICH Q8(R2) | Pharmaceutical Development |
| ICH Q9 | Quality Risk Management |
| ICH Q10 | Pharmaceutical Quality System |
| USP <1> | Injections and Implanted Drug Products (Sterility) |
| USP <71> | Sterility Tests |
| USP <85> | Bacterial Endotoxins Test |
| USP <787> | Subvisible Particulate Matter in Therapeutic Protein Injections |
| CVM-CMC-FS-001 | Cervimab DP Formulation and Manufacturing Process Description |
| CVM-CMC-SOP-001 | Aseptic Processing and Clean Room Operations |
| CVM-CMC-SOP-002 | 0.22 μm Sterilizing Filtration |
| CVM-CMC-SOP-003 | Vial Filling, Stoppering, and Capping Operations |
| CVM-CMC-QC-001 | In-Process and Release Testing Specifications for Cervimab DP |

# 3. DRUG PRODUCT FORMULATION

## 3.1 Composition

| Component | Function | Grade / Standard | Target Conc. | Quantity / 2.0 L |
| --- | --- | --- | --- | --- |
| Cervimab (Anti-PD-1 mAb) | Active | In-house biologic | 20.0 mg/mL | 40,000 mg (DS) |
| L-Histidine | Buffer | USP/NF; Ph. Eur. | 10 mM | 3.10 g |
| L-Histidine HCl Monohydrate | Buffer (pH adjust) | USP/NF; Ph. Eur. | 10 mM | 4.19 g |
| Sucrose | Stabilizer/Tonicity | USP/NF; Ph. Eur. | 8.0% (w/v) | 160.0 g |
| Polysorbate 80 (PS-80) | Surfactant | NF; Ph. Eur. | 0.04% (w/v) | 0.80 g |
| Water for Injection (WFI) | Vehicle | USP; Ph. Eur. | q.s. to volume | q.s. to 2.0 L |

## 3.2 Formulation Target Parameters

| Parameter | Target | Acceptance Criterion |
| --- | --- | --- |
| Protein Concentration | 20.0 mg/mL | 19.0 – 21.0 mg/mL |
| pH | 6.0 | 5.8 – 6.2 |
| Osmolality | ~310 mOsm/kg | 280 – 340 mOsm/kg |
| Appearance | Clear, colorless to pale yellow | Free of visible particles |
| Fill Volume | 10.5 mL nominal | 10.5 ± 0.3 mL |
| Bioburden (pre-filtration) | < 10 CFU/100 mL | Must pass |
| Endotoxin | < 0.083 EU/mg | < 0.083 EU/mg protein |

# 4. EQUIPMENT AND CRITICAL MATERIALS

## 4.1 Equipment List

| Equipment ID | Description | Size / Capacity | Qualification Status | Last Calibration |
| --- | --- | --- | --- | --- |
| EQ-001 | Stainless Steel Formulation Vessel | 3.0 L | OQ/PQ Completed | [Date] |
| EQ-002 | Overhead Mixing Assembly | Variable speed | OQ Completed | [Date] |
| EQ-003 | Peristaltic Pump (Formulation) | 0–500 mL/min | IQ/OQ Completed | [Date] |
| EQ-004 | 0.22 μm Sterilizing Filter Train | 2 x 0.22 μm PVDF | Validated | [Date] |
| EQ-005 | Filling Machine (time-pressure) | 10–500 mL/min | OQ/PQ Completed | [Date] |
| EQ-006 | Stoppering / Capping Unit | Automated | OQ/PQ Completed | [Date] |
| EQ-007 | Freeze Dryer (standby) | Not used for this batch | N/A | N/A |
| EQ-008 | Autoclave (stopper sterilization) | 200 L | Validated Cycle | [Date] |
| EQ-009 | UV-Vis Spectrophotometer | Bench-top | Calibrated | [Date] |
| EQ-010 | pH Meter | Bench-top | Calibrated | [Date] |
| EQ-011 | Osmometer | Bench-top | Calibrated | [Date] |
| EQ-012 | Particle Counter (HIAC) | Online | Calibrated | [Date] |

## 4.2 Critical Raw Materials Verification

| Material | Lot/Cert # | Vendor | CoA Verified? | Approved By | Date |
| --- | --- | --- | --- | --- | --- |
| Drug Substance (CVM-DS-P1-001) | DS-LOT-001 | In-house | Yes | ____ | [Date] |
| L-Histidine | HIS-[LOT] | Sigma Aldrich | Yes | ____ | [Date] |
| L-Histidine HCl Monohydrate | HISHCL-[LOT] | Sigma Aldrich | Yes | ____ | [Date] |
| Sucrose (injectable grade) | SUC-[LOT] | Alfa Aesar | Yes | ____ | [Date] |
| Polysorbate 80 (NF) | PS80-[LOT] | Croda | Yes | ____ | [Date] |
| WFI (in-house generated) | WFI-[LOT] | Facility | Yes | ____ | [Date] |
| 20 mL Type I glass vials | VIAL-[LOT] | Schott | Yes | ____ | [Date] |
| 20 mm FluroTec stoppers | STOP-[LOT] | West Pharma | Yes | ____ | [Date] |
| 20 mm Aluminum seals | CAP-[LOT] | West Pharma | Yes | ____ | [Date] |

# 5. ENVIRONMENTAL MONITORING AND PERSONNEL QUALIFICATION

## 5.1 Clean Room Classification

| Area | ISO Class | EU GMP Grade | Max Particles ≥0.5μm/m³ (at rest) | HVAC Status |
| --- | --- | --- | --- | --- |
| Formulation/Compounding Area | ISO 7 | Grade C (background) | 352,000 | Verified OK |
| Filling/Stoppering Zone | ISO 5 (LAF) | Grade A (in operation) | 3,520 | Verified OK |
| Stopper/Vial Staging | ISO 7 | Grade C | 352,000 | Verified OK |
| Corridor (gowning) | ISO 8 | Grade D | 3,520,000 | Verified OK |

## 5.2 Environmental Monitoring During Manufacturing

| Monitoring Point | Method | Specification | Result | Operator Initials |
| --- | --- | --- | --- | --- |
| Grade A filling zone (viable, settle plates) | Settle plates (TSA) | < 1 CFU/4h | ____ | ____ |
| Grade A filling zone (non-viable, particle) | Particle counter | ≤0 particles ≥5μm/m³ | ____ | ____ |
| Grade C compounding (viable, active air) | RCS sampler (TSA) | < 100 CFU/m³ | ____ | ____ |
| Personnel gowning qualification | Fingertip plates | < 5 CFU/glove | ____ | ____ |
| Surface monitoring (filling machine) | Contact plates | < 25 CFU/plate | ____ | ____ |
| Temperature (manufacturing area) | Calibrated probe | 18–22°C | ____°C | ____ |
| Relative Humidity | Calibrated probe | 30–60% RH | ____% | ____ |

## 5.3 Personnel Training Verification

| Personnel Role | Training Record # | Aseptic Qualified? | Gowning Qualified? | Sign-Off |
| --- | --- | --- | --- | --- |
| Batch Record Author / Formulation Operator | TR-[XXX] | Yes | Yes | ____ |
| Filling Operator | TR-[XXX] | Yes | Yes | ____ |
| Stoppering/Capping Operator | TR-[XXX] | Yes | Yes | ____ |
| QC In-Process Analyst | TR-[XXX] | Yes | Yes | ____ |
| QA Oversight | TR-[XXX] | Yes | Yes | ____ |

# 6. MANUFACTURING OPERATIONS

## 6.1 Pre-Manufacturing Checklist

| Item | Requirement | Verified? | Initials | Time |
| --- | --- | --- | --- | --- |
| 1 | Batch Manufacturing Record issued and approved by QA | Yes / No | ____ | ____ |
| 2 | All equipment identified, cleaned, and status label affixed | Yes / No | ____ | ____ |
| 3 | Previous batch clearance completed (line clearance log attached) | Yes / No | ____ | ____ |
| 4 | Environmental monitoring initiated | Yes / No | ____ | ____ |
| 5 | All raw materials identified, released, and brought to manufacturing area | Yes / No | ____ | ____ |
| 6 | Drug Substance thaw complete; DS concentration verified by UV-Vis (A280) | Yes / No | ____ | ____ |
| 7 | Vials, stoppers, and caps cleaned and sterilized per SOP (Autoclave Cycle Log Attached) | Yes / No | ____ | ____ |
| 8 | WFI quality verified (conductivity, TOC, endotoxin) | Yes / No | ____ | ____ |

## 6.2 Buffer / Excipient Solution Preparation

### Step 6.2.1 – Histidine Buffer Preparation (Target pH 6.0)

| Step # | Operation Description | Parameter / Target | Actual | Operator | Time |
| --- | --- | --- | --- | --- | --- |
| 6.2.1a | Tare formulation vessel (EQ-001). Add ~900 mL WFI. | WFI Volume = 900 mL | ___mL | ____ | ____ |
| 6.2.1b | Weigh L-Histidine; add to vessel. Mix with overhead stirrer (EQ-002) at 100 rpm. | 3.10 g ± 0.05 g | ___g | ____ | ____ |
| 6.2.1c | Weigh L-Histidine HCl Monohydrate; add to vessel. Continue mixing. | 4.19 g ± 0.05 g | ___g | ____ | ____ |
| 6.2.1d | Weigh Sucrose; add to vessel. Continue mixing at 100 rpm until dissolved. | 160.0 g ± 0.5 g | ___g | ____ | ____ |
| 6.2.1e | Weigh Polysorbate 80; add to vessel. Reduce mixing speed to 50 rpm to avoid foaming. | 0.80 g ± 0.01 g | ___g | ____ | ____ |
| 6.2.1f | Measure pH of buffer solution; adjust to 6.0 with 1N HCl or 1N NaOH if needed. | pH 6.0 ± 0.1 | ___ | ____ | ____ |
| 6.2.1g | Adjust volume to ~1.8 L with WFI; record final volume. | ~1,800 mL | ___mL | ____ | ____ |

## 6.3 Drug Product Compounding (Protein Addition)

| Step # | Operation Description | Parameter / Target | Actual | Operator | Time |
| --- | --- | --- | --- | --- | --- |
| 6.3.1 | Retrieve Drug Substance (DS) Lot CVM-DS-P1-001 from -80°C storage. Record thaw start time. | DS Lot confirmed | ____ | ____ | ____ |
| 6.3.2 | Thaw DS at 2–8°C. Confirm DS concentration by A280 UV-Vis: target ~50 mg/mL (as per DS CoA). | DS conc. verified | ___mg/mL | ____ | ____ |
| 6.3.3 | Calculate volume of DS required to achieve 20 mg/mL in 2.0 L final volume. V_DS = (20 mg/mL x 2,000 mL) / [DS conc.]. | ~800 mL (calc.) | ___mL | ____ | ____ |
| 6.3.4 | Add calculated DS volume slowly to formulation vessel (EQ-001) containing buffer. Mix at 50 rpm, no foaming. | DS added per calc. | ___mL added | ____ | ____ |
| 6.3.5 | Adjust volume to 2,000 mL with formulation buffer. Mix at 50 rpm for 15 minutes. | Final Vol = 2,000 mL | ___mL | ____ | ____ |
| 6.3.6 | Measure in-process protein concentration by A280: target 20.0 mg/mL. | 19.0–21.0 mg/mL | ___mg/mL | ____ | ____ |
| 6.3.7 | Measure pH of final formulation: target 6.0. | 5.8–6.2 | ___ | ____ | ____ |
| 6.3.8 | Measure osmolality: target ~310 mOsm/kg. | 280–340 mOsm/kg | ___mOsm/kg | ____ | ____ |
| 6.3.9 | Perform pre-filtration bioburden sample: collect 100 mL and submit to QC (Alert: < 10 CFU/100 mL). | < 10 CFU/100 mL | Submitted | ____ | ____ |

QA Review of Compounding Steps: Initials: _____ Date: _____ Time: _____

## 6.4 Sterilizing Filtration

### Step 6.4 – 0.22 μm Sterilizing Filtration (per SOP CVM-CMC-SOP-002)

| Step # | Operation Description | Parameter / Target | Actual | Operator | Time |
| --- | --- | --- | --- | --- | --- |
| 6.4.1 | Assemble sterilizing filter train (EQ-004): 2 x 0.22 μm PVDF Millipak/Durapore filters in series. Confirm filter lot numbers and certificate of conformance. | Filter integrity verified | ____ | ____ | ____ |
| 6.4.2 | Perform pre-use integrity test on upstream filter (Forward Flow / Diffusive Flow test). Record result. | < 15 mL/min @ 10 psi | ___mL/min | ____ | ____ |
| 6.4.3 | Perform pre-use integrity test on downstream filter. | < 15 mL/min @ 10 psi | ___mL/min | ____ | ____ |
| 6.4.4 | Filter bulk formulation through sterilizing filter train into sterile holding vessel (pre-sterilized by autoclave). Record flow rate. | Positive flow, no bypass | ___mL/min | ____ | ____ |
| 6.4.5 | After filtration, perform post-use integrity test on both filters. Both must pass. | Both pass Forward Flow | ____ | ____ | ____ |
| 6.4.6 | Record total filtered volume. | ~2,000 mL | ___mL | ____ | ____ |
| 6.4.7 | Measure post-filtration protein concentration by A280. | 19.0–21.0 mg/mL | ___mg/mL | ____ | ____ |
| 6.4.8 | Collect 100 mL post-filtration bioburden sample. Submit to QC. | < 1 CFU/100 mL | Submitted | ____ | ____ |

CRITICAL STEP: Both pre- and post-use filter integrity tests must PASS before proceeding. Failure of any filter requires batch investigation and QA review prior to continuation.

## 6.5 Vial Filling, Stoppering, and Capping

| Step # | Operation Description | Parameter / Target | Actual | Operator | Time |
| --- | --- | --- | --- | --- | --- |
| 6.5.1 | Set up filling machine (EQ-005) under Grade A laminar airflow. Confirm fill volume calibration: target 10.5 mL/vial. | 10.5 ± 0.3 mL | ___mL | ____ | ____ |
| 6.5.2 | Perform initial weight check on first 10 vials (manual weigh). Accept if all within spec; adjust fill pump if needed. | 10.5 ± 0.3 mL | ____ | ____ | ____ |
| 6.5.3 | Begin filling. Perform in-process fill weight checks every 30 minutes (n=10 vials each check). Record results. | 10.5 ± 0.3 mL | ____ | ____ | ____ |
| 6.5.4 | Perform stopper application via automated stoppering unit (EQ-006) immediately after filling under Grade A. Confirm 100% stoppering. | 100% stoppered | ____ | ____ | ____ |
| 6.5.5 | Transfer stopperd vials to capping station; apply aluminum seal (EQ-006). | 100% capped | ____ | ____ | ____ |
| 6.5.6 | Record total vials filled, total vials rejected (visible particulates, fill volume, damaged seals). | Target: 200 vials | ____filled, ____rejected | ____ | ____ |
| 6.5.7 | Collect first 3 and last 3 vials of fill as reference samples. | 6 vials retained | ____ | ____ | ____ |
| 6.5.8 | Collect 20 vials for 100% visual inspection (appearance, fill level, seal integrity). | 100% pass | ____ | ____ | ____ |

## 6.6 In-Process Fill Weight Log

| Time Point | Vial #1 (g) | Vial #2 (g) | Vial #3 (g) | Vial #4 (g) | Vial #5 (g) | Mean (g) | Pass/Fail | Operator |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Start (T=0) | ____ | ____ | ____ | ____ | ____ | ____ | ____ | ____ |
| T = 30 min | ____ | ____ | ____ | ____ | ____ | ____ | ____ | ____ |
| T = 60 min | ____ | ____ | ____ | ____ | ____ | ____ | ____ | ____ |
| T = 90 min | ____ | ____ | ____ | ____ | ____ | ____ | ____ | ____ |
| T = 120 min | ____ | ____ | ____ | ____ | ____ | ____ | ____ | ____ |
| End of Fill | ____ | ____ | ____ | ____ | ____ | ____ | ____ | ____ |

Fill weight acceptance criterion: Net fill weight = 10.80 g ± 0.31 g (based on solution density ~1.029 g/mL at 20°C)

# 7. IN-PROCESS CONTROLS AND RELEASE TESTING SUMMARY

## 7.1 In-Process Control Results

| Test | Specification | Timing | Result | Pass/Fail | Analyst Initials |
| --- | --- | --- | --- | --- | --- |
| Protein concentration (A280) | 19.0–21.0 mg/mL | Post-compounding | ____ | ____ | ____ |
| pH | 5.8–6.2 | Post-compounding | ____ | ____ | ____ |
| Osmolality | 280–340 mOsm/kg | Post-compounding | ____ | ____ | ____ |
| Pre-filtration bioburden | < 10 CFU/100 mL | Pre-filtration | ____ | ____ | ____ |
| Post-filtration bioburden | < 1 CFU/100 mL | Post-filtration | ____ | ____ | ____ |
| Filter integrity (pre-use) | Both filters pass | Pre-filtration | ____ | ____ | ____ |
| Filter integrity (post-use) | Both filters pass | Post-filtration | ____ | ____ | ____ |
| Fill volume (gravimetric) | 10.5 ± 0.3 mL | During filling | ____ | ____ | ____ |
| Appearance (in-process) | Clear, colorless, particle-free | During filling | ____ | ____ | ____ |

## 7.2 Batch Release Testing

The following release tests are performed by QC on representative samples from this batch per CVM-CMC-QC-001. Results must meet all specifications prior to release for clinical use.

| Test | Method | Specification | Result | Pass/Fail |
| --- | --- | --- | --- | --- |
| Identity (Protein A ELISA, SPR) | In-house validated | Anti-PD-1 confirmed | Pending | ____ |
| Protein Concentration (A280) | UV-Vis spectrophotometry | 19.0–21.0 mg/mL | Pending | ____ |
| Purity (SEC-HPLC) | Size Exclusion Chromatography | ≥ 97% monomer | Pending | ____ |
| Purity (CEX-HPLC charge variants) | Cation Exchange Chromatography | Report result | Pending | ____ |
| Aggregates (DLS) | Dynamic Light Scattering | ≤ 1.0% aggregate | Pending | ____ |
| Potency (Cell-based PD-1 binding) | In-house validated bioassay | 70–130% relative potency | Pending | ____ |
| pH | Calibrated pH meter | 5.8–6.2 | Pending | ____ |
| Osmolality | Vapor pressure osmometer | 280–340 mOsm/kg | Pending | ____ |
| Appearance | Visual inspection under white/black background | Clear, colorless to pale yellow | Pending | ____ |
| Visible Particulates | 100% visual inspection | Essentially free | Pending | ____ |
| Subvisible Particles (HIAC) | USP <787> | ≤25/mL (≥1μm); ≤3/mL (≥10μm) | Pending | ____ |
| Sterility | USP <71> direct inoculation / membrane filtration | No growth at 14 days | Pending | ____ |
| Bacterial Endotoxins (BET) | USP <85> LAL gel-clot/kinetic | < 0.083 EU/mg | Pending | ____ |
| Container Closure Integrity (CCI) | Vacuum decay or dye ingress | No dye ingress / pass | Pending | ____ |
| Extractables (PS-80 content) | LC-MS or colorimetric | 0.03–0.05% w/v | Pending | ____ |

# 8. DEVIATIONS, INVESTIGATIONS, AND COMMENTS

| Deviation # | Step # | Description of Deviation | Impact Assessment | Corrective Action | QA Approval |
| --- | --- | --- | --- | --- | --- |
| None recorded | — | — | — | — | — |

Note: Any deviation from this Batch Record must be documented in the Deviation/CAPA system prior to batch disposition. Deviations are assigned a risk classification (Minor, Major, Critical) per SOP CVM-QA-DEV-001.

# 9. YIELD RECONCILIATION AND MATERIAL ACCOUNTABILITY

| Parameter | Calculation | Value |
| --- | --- | --- |
| Theoretical Yield | 200 vials x 10.5 mL = 2,100 mL gross | 2,100 mL / 200 vials |
| Actual Vials Filled | As recorded in Step 6.5.6 | ____ vials |
| Vials Rejected (QC) | As recorded in Step 6.5.6 | ____ vials |
| Actual Release Vials | Filled – Rejected – Samples | ____ vials |
| % Yield (units) | (Actual Release / Theoretical) x 100 | ____% |
| Protein Mass Input (DS) | DS Vol (mL) x DS Conc (mg/mL) | ____ mg |
| Protein Mass Output (DP) | DP vials x fill vol x DP conc | ____ mg |
| % Protein Yield | (Mass Output / Mass Input) x 100 | ____% (Limit: ≥ 85%) |

# 10. STORAGE, LABELING, AND BATCH DISPOSITION

| Item | Details |
| --- | --- |
| Storage Condition | 2–8°C (refrigerated), protected from light |
| Storage Location | Clinical Investigational Product Storage Room [Room #] |
| Clinical Label Applied? | Yes □ No □ (INVESTIGATIONAL USE ONLY label, 21 CFR 312.6) |
| Quarantine Status | QUARANTINE until all release testing complete and QA disposition issued |
| QA Batch Disposition | Released □ Rejected □ Conditional Release □ |
| QA Authorized Signatory | _________________________ Date: _______ |
| Expiry / Retest Date | [TBD per stability protocol CVM-CMC-STA-001 — provisional: 18 months at 2–8°C] |
| Distribution Record # | To be completed upon clinical site shipment (per CVM-IRT-001) |

# 11. APPENDICES AND ATTACHED RECORDS

| Appendix | Document | Attached? |
| --- | --- | --- |
| A | Autoclave Cycle Log (Stopper/Vial Sterilization) | Yes □ / N/A □ |
| B | WFI Quality Log (Batch Day) | Yes □ / N/A □ |
| C | Environmental Monitoring Results (Settlement plates, active air, surface) | Yes □ / N/A □ |
| D | Equipment Calibration Certificates (pH meter, balance, spectrophotometer) | Yes □ / N/A □ |
| E | Filter Integrity Test Certificates (pre- and post-use) | Yes □ / N/A □ |
| F | Drug Substance CoA (Lot CVM-DS-P1-001) | Yes □ / N/A □ |
| G | Raw Material CoAs (Histidine, Sucrose, PS-80, WFI, Vials, Stoppers, Caps) | Yes □ / N/A □ |
| H | In-Process Control Lab Worksheets | Yes □ / N/A □ |
| I | Visual Inspection Log | Yes □ / N/A □ |
| J | Sample Chain of Custody Forms | Yes □ / N/A □ |
| K | Deviation Reports (if applicable) | Yes □ / N/A □ |
| L | Batch Release Certificate (QA) | Yes □ / N/A □ |

# 12. REVIEW AND APPROVAL SIGNATURES

| Role / Title | Printed Name / Signature | Date |
| --- | --- | --- |
| Manufacturing Operator (Lead) | ___________________________ | ___________ |
| Manufacturing Operator (Secondary) | ___________________________ | ___________ |
| QC In-Process Analyst | ___________________________ | ___________ |
| Manufacturing Manager | ___________________________ | ___________ |
| QA Reviewer | ___________________________ | ___________ |
| QA Authorized Person (Batch Release) | ___________________________ | ___________ |

END OF BATCH MANUFACTURING RECORD  —  CVM-CMC-BR-001  —  EXAMPLE DOCUMENT ONLY

This document is provided for illustrative and educational purposes only. It does not represent an actual product, actual manufacturing process, or regulatory submission. All data fields, lot numbers, and personnel identifiers are fictional.