COMPARABILITY PROTOCOL

Cervimab (CVM-001) - Anti-PD-1 IgG4 Monoclonal Antibody

Manufacturing Process Change: Phase 3 to Commercial Scale

Pre-Change (Phase 3) vs. Post-Change (Commercial PPQ) Drug Substance

| Protocol No. | CMP-001 |
| --- | --- |
| Molecule | Cervimab (CVM-001) |
| Change Description | Scale-up: 200 L (Phase 3) to 2,000 L (commercial) bioreactor; introduction of single-use bioreactor technology at N-1 seed stage; updated UF/DF membrane module |
| Pre-Change Material | Phase 3 clinical batches: CVM-DS-P3-001, P3-002, P3-003 (200 L scale) |
| Post-Change Material | Commercial PPQ batches: CVM-DS-PPQ-V01, V02, V03 (2,000 L scale) |
| Regulatory Pathway | ICH Q5E - Comparability of Biotechnological/Biological Products; FDA Guidance on Comparability (1996); EMA CHMP Guideline on Comparability |
| Conclusion Required By | Prior to commercial launch; submission with BLA [pending] |
| Document Status | EXAMPLE DOCUMENT - ILLUSTRATIVE PURPOSES ONLY |

# 1. INTRODUCTION AND BACKGROUND

## 1.1 Purpose

This Comparability Protocol (CMP-001) defines the criteria and analytical testing strategy to assess whether cervimab drug substance manufactured at the commercial scale (2,000 L fed-batch bioreactor, Site A) is comparable to drug substance manufactured at the Phase 3 clinical scale (200 L fed-batch bioreactor, Site A) used in the pivotal Phase 3 clinical trial (Study CVM-301). The objective is to demonstrate that the manufacturing process change has not adversely affected the safety, efficacy, or quality of cervimab.

## 1.2 Description of Manufacturing Changes

Three manufacturing changes were introduced between Phase 3 and commercial-scale manufacturing:

| No. | Change | Phase 3 (Pre-Change) | Commercial (Post-Change) |
| --- | --- | --- | --- |
| 1 | Production Bioreactor Scale | 200 L stainless steel stirred tank | 2,000 L stainless steel stirred tank (10x scale-up) |
| 2 | N-1 Seed Bioreactor | 20 L stainless steel, N-1 stage | 200 L single-use bioreactor (Thermo Fisher HyPerforma S.U.B.) |
| 3 | UF/DF Module | 0.1 m2 Millipore Pellicon 3, 30 kDa MWCO | 1.0 m2 Millipore Pellicon 3, 30 kDa MWCO (same membrane material, larger cassette format) |

The formulation, downstream purification chemistry, and container closure system are UNCHANGED between Phase 3 and commercial manufacturing. The production bioreactor process conditions (temperature, pH, DO, nutrient feeds, culture duration) are within the established proven acceptable ranges (PARs) and characterized design space.

## 1.3 Regulatory Context

Under ICH Q5E, the purpose of comparability assessment is to establish that the pre- and post-change product has a similar profile with respect to quality, safety, and efficacy. If comparability is established at the quality level, clinical bridging is not required. This protocol defines the approach to demonstrating quality comparability. Safety and efficacy comparability are supported by the unchanged clinical indication, dosing regimen, and formulation.

# 2. COMPARABILITY TESTING STRATEGY

## 2.1 Tiered Testing Approach

A risk-based, tiered testing approach is employed, consistent with ICH Q5E and ICH Q8(R2). Testing is categorized into three tiers based on the criticality of the attribute to clinical safety and efficacy:

| Tier | Category | Description | Statistical Criterion |
| --- | --- | --- | --- |
| 1 | Critical / High Risk | Attributes with direct impact on safety or efficacy (potency, aggregates, glycosylation pattern) | Equivalence test (90% CI within acceptance margin) or individual value comparison vs. specification |
| 2 | Important / Medium Risk | Attributes important for product consistency but with indirect clinical impact (charge variants, HCP, process impurities) | Statistical comparison (two-sample t-test or Mann-Whitney, p < 0.05) and within-specification assessment |
| 3 | Supportive / Low Risk | Attributes providing structural confirmation or supporting data (CD spectroscopy, DSC, higher-order structure) | Qualitative assessment; visual comparison; overlay of profiles |

## 2.2 Batch Disposition for Comparability

Pre-change: Three Phase 3 clinical-scale drug substance batches (CVM-DS-P3-001, P3-002, P3-003), manufactured at 200 L scale under the Phase 3 process.

Post-change: Three commercial-scale PPQ drug substance batches (CVM-DS-PPQ-V01, V02, V03), manufactured at 2,000 L scale.

All six batches were manufactured at the same site (Acme Biologics, RTP) using the same CHO-K1 cell bank (MCB-Gen3-Lot001). Side-by-side testing was conducted from retained samples stored at <=−70°C.

# 3. ANALYTICAL TESTING PLAN

## 3.1 Tier 1 - Critical Attributes (Full Comparability Required)

| Attribute | Method | Acceptance Criterion | Statistical Test | Margin |
| --- | --- | --- | --- | --- |
| Biological Potency (PD-1/PD-L1 blockade, EC50) | Cell-based reporter assay | 90% CI of post/pre ratio within 0.80-1.25 | Equivalence test | +/-25% |
| Purity (SEC-HPLC, % monomer) | SEC-HPLC | 90% CI of difference within +/-1.5% | Equivalence test | +/-1.5% |
| N-Glycan Profile (major species: G0F, G1F, G2F, high mannose) | HILIC-FLD + LC-MS | Individual glycan species: diff <= 10% (absolute) | Descriptive comparison | +/-10% abs. |
| Aggregates (SEC-HPLC, % HMW) | SEC-HPLC | 90% CI of difference within +/-0.5% | Equivalence test | +/-0.5% |
| Sub-visible Particles (MFI, >=2 um and >=10 um/mL) | MFI | Individual batch values within specification; no statistically significant increase | Comparison to spec + t-test | Within spec. |
| Binding Affinity to PD-1 (KD, nM, SPR) | SPR (Biacore T200) | Ratio of post/pre means: 0.75-1.33 | Ratio comparison | +/-33% |

## 3.2 Tier 2 - Important Attributes (Statistical Comparability Required)

| Attribute | Method | Acceptance Criterion | Statistical Test |
| --- | --- | --- | --- |
| Charge Variant Profile (cIEF - main peak, acidic, basic %) | cIEF (iCE3) | No statistically significant difference; all values within spec | Two-sample t-test (alpha=0.05) |
| HCP (ppm) | CHO HCP ELISA | No significant increase; within spec | Two-sample t-test |
| Host Cell DNA (pg/mg) | qPCR | Both pre and post within spec; no significant increase | Comparison to specification |
| Deamidation (Asn-55 HC, % by peptide mapping) | Peptide mapping (LC-MS/MS) | Diff <= 3% absolute; within spec | Descriptive + within spec |
| Oxidation (Met-254 HC, % by peptide mapping) | Peptide mapping (LC-MS/MS) | Diff <= 2% absolute; within spec | Descriptive + within spec |
| Polysorbate 20 content | HPLC-CAD | Within specification | Comparison to spec |
| Endotoxin (EU/mg) | LAL USP <85> | Within specification (<=1.0 EU/mg) | Comparison to spec |

## 3.3 Tier 3 - Supportive Structural Characterization

| Attribute | Method | Comparability Assessment |
| --- | --- | --- |
| Higher-Order Structure (secondary, tertiary) | CD spectroscopy + HDX-MS | Overlay of CD spectra (190-260 nm); visual comparison; no significant shift in secondary structure content |
| Thermal Stability (Tm1, Tm2) | DSC (nano-DSC) | Tm within +/-2 deg C; thermogram profile qualitatively similar |
| Hydrodynamic Radius (Rh) by DLS | DLS | Mean Rh within +/-0.5 nm; PDI comparable |
| Intact Mass (non-reduced, deglycosylated) | LC-MS (intact mass) | Mass difference <= 5 Da from theoretical; consistent with expected primary sequence |
| Disulfide Bond Integrity | Peptide mapping (non-reduced) | All 16 disulfide bonds confirmed; no aberrant free thiols |
| FcRn Binding (pH 6.0 and 7.4) | SPR (Biacore) | KD ratio (post/pre) within 0.67-1.50 at pH 6.0; qualitative similarity at pH 7.4 |

# 4. PRE-DEFINED ACCEPTANCE CRITERIA AND DECISION TREE

## 4.1 Overall Comparability Conclusion Criteria

Comparability is established (product is 'comparable') when ALL of the following conditions are met:

- ALL Tier 1 critical attributes meet their pre-defined acceptance criteria

- ALL Tier 2 important attributes show no statistically significant differences AND all individual values are within the approved specification

- ALL Tier 3 supportive attributes show qualitative similarity with no unexpected differences

- No new impurities are identified in the post-change material above reporting thresholds

- No new structural variants are identified by mass spectrometry that are absent from the pre-change material

If ANY Tier 1 criterion is not met, comparability is not established and a clinical bridging study may be required.

If any Tier 2 or Tier 3 finding is unexpected, a root cause investigation will be conducted and the impact on product quality and clinical safety/efficacy will be assessed prior to a final comparability determination.

## 4.2 Decision Tree

| Scenario | Action Required | Regulatory Consequence |
| --- | --- | --- |
| All criteria met (Tier 1, 2, 3) | Issue Comparability Report (CRM-001); proceed to commercial launch | No clinical bridging study required; document in BLA Module 3 |
| Tier 1 failure (1 attribute) | Root cause investigation; risk assessment; CAPA; consider clinical bridging | May require clinical bridging study; FDA/EMA consultation |
| Tier 2/3 failure only | Root cause investigation; risk assessment; documented justification for acceptable difference | Additional data package in BLA; no clinical bridging if justified |

# 5. COMPARABILITY ASSESSMENT RESULTS

## 5.1 Summary of Comparability Findings

Side-by-side testing of three Phase 3 pre-change batches vs. three commercial PPQ post-change batches was conducted. The following table summarizes the key results. Full data tabulations and statistical outputs are provided in the Comparability Report (CRM-001).

| Attribute | Tier | Pre-Change Mean (n=3) | Post-Change Mean (n=3) | Statistical Result | Comparable? |
| --- | --- | --- | --- | --- | --- |
| Potency (% RS, EC50) | 1 | 99.3% | 101% | 90% CI: 0.91-1.11; within 0.80-1.25 | YES |
| Monomer % (SEC-HPLC) | 1 | 98.8% | 99.1% | 90% CI diff: -0.4% to +0.9%; within +/-1.5% | YES |
| KD PD-1 (nM, SPR) | 1 | 0.84 nM | 0.81 nM | Ratio 0.96; within 0.75-1.33 | YES |
| HMW aggregates (SEC, %) | 1 | 0.51% | 0.48% | 90% CI diff: -0.06% to +0.09%; within +/-0.5% | YES |
| G0F glycan (HILIC, %) | 1 | 44.2% | 45.1% | Diff = 0.9% abs; within +/-10% | YES |
| High Mannose (HILIC, %) | 1 | 3.8% | 4.1% | Diff = 0.3% abs; within +/-10% | YES |
| HCP (ppm) | 2 | 13.1 | 8.2 | t-test p=0.08; both within spec (<=25 ppm); decrease consistent with scale-up improvement | YES |
| Acidic variants (cIEF, %) | 2 | 9.2% | 8.2% | t-test p=0.12; both within spec | YES |
| Deamidation Asn-55 HC (%) | 2 | 2.9% | 2.7% | Diff 0.2% abs; within spec and margin | YES |
| Tm1 (DSC, deg C) | 3 | 72.1 | 72.4 | Diff 0.3 deg C; within +/-2 deg C | YES |
| CD spectrum (secondary structure) | 3 | Ref overlay | Overlay | Spectra superimposable; no shift in minima | YES |
| Intact mass (Da, deglycosylated) | 3 | 146,482 Da | 146,481 Da | Diff 1 Da; within 5 Da criterion | YES |

## 5.2 New Impurities Assessment

No new product-related or process-related impurities were identified in the post-change (commercial PPQ) material above ICH Q6B reporting thresholds. Mass spectrometric peptide mapping (n=3 batches each) confirmed identical variant profiles between pre- and post-change material. No new HCP species were identified by 2D-DIGE or mass spectrometry-based HCP characterization in the post-change material.

## 5.3 Comparability Conclusion

Based on the totality of the analytical evidence, cervimab drug substance manufactured at the commercial scale (2,000 L fed-batch, Acme Biologics) is COMPARABLE to the Phase 3 clinical material (200 L scale) used in the pivotal Phase 3 trial (CVM-301). All Tier 1, Tier 2, and Tier 3 acceptance criteria are satisfied. A minor, statistically non-significant improvement in HCP clearance was observed at commercial scale, which is attributed to the improved process control at larger scale and is not considered an adverse change.

No clinical bridging study is required. This comparability package, including the full Comparability Report (CRM-001), will be submitted as part of the BLA Module 3.

# 6. IMPACT ON CLINICAL SAFETY AND EFFICACY

The comparability assessment supports the conclusion that the manufacturing process change has not adversely affected the safety or efficacy of cervimab:

- Biological potency (PD-1/PD-L1 blockade) is comparable, supporting equivalent in vivo activity

- No change in PD-1 binding affinity (KD) was observed, confirming the target engagement profile is unchanged

- Glycosylation profile is comparable, supporting equivalent Fc effector function (minimal ADCC, consistent FcRn binding)

- Aggregate and particle content is comparable or improved, supporting equivalent immunogenicity risk

- Process impurity levels (HCP, DNA) are within specifications with a non-adverse decrease at commercial scale

- Higher-order structure (CD, DSC, HDX-MS, intact mass) is indistinguishable between pre- and post-change material

These data, in conjunction with the established pharmacological mechanism of PD-1 inhibition, provide high confidence that clinical outcomes observed in Study CVM-301 are fully applicable to cervimab manufactured at the commercial scale.

# 7. APPROVALS

| Role | Name / Title | Signature / Date |
| --- | --- | --- |
| Protocol Author | Director, Drug Substance Development, CerviGen Therapeutics | _________________________ |
| Reviewed By | Head of Analytical Development, CerviGen Therapeutics | _________________________ |
| Reviewed By | Vice President, Regulatory Affairs (CMC), CerviGen Therapeutics | _________________________ |
| Approved By | Chief Scientific Officer, CerviGen Therapeutics | _________________________ |