PROCESS VALIDATION REPORT

Cervimab Drug Substance - Commercial Manufacturing

Process Performance Qualification (PPQ) - Stages 1 & 2

| Report No. | PVR-001 |
| --- | --- |
| Drug Substance | Cervimab (CVM-001) - Anti-PD-1 IgG4 Monoclonal Antibody |
| Manufacturing Site | Acme Biologics Manufacturing Inc., Research Triangle Park, NC 27709 |
| Process Scale | 2,000 L fed-batch bioreactor (commercial scale) |
| PPQ Batches | CVM-DS-PPQ-V01, CVM-DS-PPQ-V02, CVM-DS-PPQ-V03 |
| PPQ Execution Period | January 2023 - June 2023 |
| Regulatory Framework | FDA Process Validation Guidance (2011); ICH Q8(R2), Q9, Q10; 21 CFR 211 |
| Prepared By | Process Validation Team, CerviGen Therapeutics / Acme Biologics |
| Document Status | EXAMPLE DOCUMENT - ILLUSTRATIVE PURPOSES ONLY |

# 1. PURPOSE AND SCOPE

This Process Validation Report (PVR-001) documents the execution and results of the Process Performance Qualification (PPQ) for the commercial manufacturing process of cervimab drug substance at Acme Biologics Manufacturing Inc. This report fulfills Stage 2 of the three-stage process validation lifecycle as defined in the FDA Guidance for Industry: Process Validation: General Principles and Practices (January 2011).

The PPQ demonstrates that the commercial-scale manufacturing process consistently produces cervimab drug substance of predefined quality, consistent with results from process development and Stage 1 (process design) activities documented in the Process Design Report (PDR-001) and Process Characterization Report (PCR-001).

The scope of this report encompasses the following process operations performed at the commercial scale (2,000 L bioreactor):

- Cell banking through 2,000 L fed-batch production bioreactor

- Harvest, clarification, and purification (Protein A, Low pH VI, CEX, AEX, nanofiltration)

- UF/DF concentration and formulation

- Bulk drug substance sterile filtration, fill, and storage

Stage 3 (Continued Process Verification, CPV) is described in the CPV Protocol (CPVP-001) and is ongoing.

# 2. REGULATORY AND SCIENTIFIC BASIS

| Regulatory Reference | Relevance to This Validation |
| --- | --- |
| FDA Process Validation Guidance, Jan 2011 | Three-stage lifecycle framework; PPQ design and execution; CPV requirements |
| ICH Q8(R2) - Pharmaceutical Development | Design space; enhanced development approach; QbD elements |
| ICH Q9 - Quality Risk Management | Risk-based approach to identifying CQAs and CPPs; PPQ acceptance criteria |
| ICH Q10 - Pharmaceutical Quality System | Process performance monitoring; management review; CAPA |
| ICH Q5A(R2) - Viral Safety | Viral clearance validation incorporated into process validation |
| 21 CFR Part 211 - cGMP for Finished Pharmaceuticals | Equipment qualification, written procedures, batch record review |
| EMA/CHMP/BWP/187338/2014 - Biotech Process Validation | EU requirements for biologics PPQ, CPV, and lifecycle management |

# 3. PPQ DESIGN AND STRATEGY

## 3.1 PPQ Strategy Overview

The PPQ strategy was developed based on a comprehensive process characterization program (PCR-001) that identified Critical Quality Attributes (CQAs), Critical Process Parameters (CPPs), and their proven acceptable ranges (PARs). The PPQ protocol (PVPR-001) defined:

- Number of PPQ batches: 3 consecutive commercial-scale batches (consistent with FDA guidance and industry precedent for biologics)

- Acceptance criteria: predefined, scientifically justified, based on process characterization data and Phase 3 clinical batch historical data

- Sampling and testing plan: enhanced testing at critical steps; full release testing of bulk DS

- Statistical approach: process capability (Ppk) analysis where applicable; graphical trend analysis

- Predetermined success criteria per ICH Q8(R2) design space

## 3.2 Critical Quality Attributes (CQAs) and Critical Process Parameters (CPPs)

CQAs and CPPs were identified through a structured risk assessment (Failure Mode and Effects Analysis, FMEA) and confirmed by multivariate process characterization studies (Design of Experiments, DOE). Key CQAs monitored during PPQ:

| CQA | Category | Method | PPQ Acceptance Criterion | Risk Tier |
| --- | --- | --- | --- | --- |
| Purity (monomer, SEC-HPLC) | Purity | SEC-HPLC | >=98.5% | High |
| Potency (PD-1/PD-L1 blockade) | Efficacy | Cell-based assay | 80-120% RS | High |
| HCP (Host Cell Protein) | Safety/Purity | CHO HCP ELISA | <=15 ppm | High |
| Host Cell DNA | Safety | qPCR | <=5 pg/mg | High |
| Leached Protein A | Process Impurity | ELISA | <=1 ppm | Medium |
| Charge variants (acidic, cIEF) | Product variant | cIEF | Acidic <=12% | Medium |
| Sub-visible particles (MFI) | Safety/Purity | MFI | >=2um: <=5000/mL | Medium |
| Concentration (A280) | Assay | UV-Vis | 19.0-21.0 mg/mL | Medium |
| Bioburden (pre-final filtration) | Microbiological | USP <61> | <=10 CFU/100 mL | High |
| Endotoxin (bulk DS) | Safety | LAL USP <85> | <=0.5 EU/mg | High |

# 4. PPQ EXECUTION - RESULTS SUMMARY

## 4.1 PPQ Batch Summary

| CQA / Attribute | Spec Criterion | PPQ V01 | PPQ V02 | PPQ V03 | All Pass? | Ppk (n=9) |
| --- | --- | --- | --- | --- | --- | --- |
| Titer (g/L, harvest) | >=2.0 | 2.48 | 2.61 | 2.53 | YES | -- |
| Monomer % (SEC-HPLC) | >=98.5% | 99.1% | 99.3% | 99.0% | YES | 2.18 |
| Potency (% RS) | 80-120% | 99% | 103% | 101% | YES | 2.41 |
| HCP (ppm) | <=15 ppm | 7.9 | 8.5 | 7.3 | YES | 2.05 |
| Host Cell DNA (pg/mg) | <=5 | 0.8 | 1.1 | 0.9 | YES | -- |
| Protein A leach (ppm) | <=1.0 | <0.1 | <0.1 | <0.1 | YES | -- |
| Acidic variants (cIEF, %) | <=12% | 8.1% | 8.4% | 8.2% | YES | 1.98 |
| Concentration (mg/mL) | 19.0-21.0 | 20.0 | 19.9 | 20.2 | YES | 2.62 |
| Bioburden (CFU/100mL) | <=10 | <1 | <1 | <1 | YES | -- |
| Endotoxin (EU/mg) | <=0.5 | <0.1 | <0.1 | <0.1 | YES | -- |

Note: Ppk calculated using n=3 PPQ + 6 Phase 3 clinical batches (total n=9). Ppk >= 1.67 target for high-risk CQAs. All applicable CQAs demonstrated Ppk >= 1.67.

## 4.2 Bioreactor Performance (2,000 L PPQ)

| Parameter | Target / PAR | PPQ V01 | PPQ V02 | PPQ V03 | Pass |
| --- | --- | --- | --- | --- | --- |
| Culture Duration (days) | 12-14 | 13 | 13 | 14 | Y |
| Peak Viable Cell Density (x10e6 cells/mL) | 18-28 | 24.1 | 23.7 | 25.3 | Y |
| Final Viability (%) | >=65% | 71.2% | 68.8% | 70.1% | Y |
| Harvest Titer (g/L) | >=2.0 | 2.48 | 2.61 | 2.53 | Y |
| Dissolved Oxygen (% setpoint) | 40 +/- 5% | 40.2% | 39.9% | 40.5% | Y |
| pH (mean culture pH) | 7.0 +/- 0.2 | 7.02 | 6.98 | 7.01 | Y |
| Temperature (mean, deg C) | 37 +/- 0.5 | 37.0 | 37.1 | 36.9 | Y |

## 4.3 Purification Step Performance

| Purification Step | Key IPC / Outcome | PPQ V01 | PPQ V02 | PPQ V03 |
| --- | --- | --- | --- | --- |
| Protein A Affinity | Purity (SEC-HPLC, % monomer) | 97.2% | 97.5% | 97.1% |
| Protein A Affinity | Leached Protein A (ppm) | 3.2 | 2.9 | 3.4 |
| Low pH VI (pH 3.5, 60 min) | Product recovery (A280, %) | 98.1% | 97.8% | 98.4% |
| CEX Chromatography | Acidic variants post-CEX (%) | 8.1% | 8.4% | 8.2% |
| CEX Chromatography | HCP post-CEX (ppm) | 11.2 | 12.1 | 10.8 |
| AEX (flow-through) | HCP post-AEX (ppm) | 8.1 | 8.8 | 7.8 |
| Nanofiltration (20 nm) | Recovery (%); filter integrity | 97.4%; Pass | 98.0%; Pass | 97.7%; Pass |
| UF/DF | Target concentration (mg/mL post-UF/DF) | 20.0 | 19.9 | 20.2 |
| Final Bulk DS | HCP final (ppm) | 7.9 | 8.5 | 7.3 |

## 4.4 Viral Clearance Summary

Viral clearance validation was performed in accordance with ICH Q5A(R2). Four dedicated clearance steps were evaluated. Total clearance demonstrated is summarized below:

| Clearance Step | Model Virus | Virus Type | Log10 Reduction | Acceptance Criterion | Pass |
| --- | --- | --- | --- | --- | --- |
| Protein A Affinity | MuLV (xenotropic) | Enveloped, ssRNA | >=4.2 log10 | >=4.0 | Y |
| Low pH Viral Inactivation | MuLV | Enveloped | >=5.6 log10 | >=4.0 | Y |
| Low pH Viral Inactivation | X-MuLV | Enveloped | >=5.1 log10 | >=4.0 | Y |
| AEX (flow-through) | MMV (parvovirus model) | Non-enveloped, ssDNA | >=2.8 log10 | >=1.0 (contribution) | Y |
| Nanofiltration (20 nm) | PPV (parvovirus model) | Non-enveloped | >=4.5 log10 | >=4.0 | Y |
| Nanofiltration (20 nm) | Reo-3 (rota model) | Non-enveloped, dsRNA | >=3.8 log10 | >=1.0 (contribution) | Y |

Overall virus safety margin: >= 9.8 log10 for enveloped viruses; >= 7.3 log10 for non-enveloped small viruses. These margins are consistent with ICH Q5A(R2) and comparable to approved PD-1 inhibitor biologics.

# 5. PROCESS ROBUSTNESS AND DEVIATIONS

## 5.1 PPQ Deviations

| Dev. No. | Batch | Description | Impact Assessment | Resolution |
| --- | --- | --- | --- | --- |
| DEV-V01-001 | PPQ V01 | DO probe sensor drift at day 7 of culture; corrected within 2 hours by manual adjustment | DO maintained within PAR; no impact on titer or product quality attributes | Closed; sensor replaced |
| DEV-V02-001 | PPQ V02 | Protein A column backpressure transiently exceeded upper alert limit at 92% of purification cycle | Step completed successfully; eluate met all IPC criteria; no quality impact | Closed; column repacked |

No deviations resulted in product failure or required batch disposition. Both deviations were classified as minor and closed without change to the process or batch record.

# 6. CONCLUSIONS AND PPQ SUMMARY STATEMENT

All three PPQ batches (CVM-DS-PPQ-V01, V02, V03) met all predefined acceptance criteria as defined in the PPQ Protocol (PVPR-001). The commercial-scale manufacturing process for cervimab drug substance at Acme Biologics Manufacturing Inc. is validated and capable of reproducibly producing drug substance that meets all quality specifications.

Key conclusions:

- All CQA acceptance criteria met across all three PPQ batches

- Process capability (Ppk) >= 1.67 demonstrated for all quantitative CQAs with sufficient data (n >= 9)

- Process robustness confirmed across defined CPP ranges; no out-of-specification results

- Viral clearance >= 4 log10 demonstrated for all relevant virus classes per ICH Q5A(R2)

- Minor deviations observed during PPQ; none impacted product quality; all formally closed

- Stage 3 Continued Process Verification (CPV) program (CPVP-001) is in place and ongoing

This report, together with the Analytical Method Validation Reports and Process Characterization Report (PCR-001), supports the regulatory submission for cervimab BLA in the indication of dMMR/MSI-H metastatic colorectal cancer.

# 7. APPROVALS

| Role | Name / Title | Signature / Date |
| --- | --- | --- |
| Prepared By | Head of Process Validation, CerviGen Therapeutics | _________________________ |
| Reviewed By | VP Manufacturing Science, Acme Biologics | _________________________ |
| Reviewed By | Head of Quality Assurance, Acme Biologics | _________________________ |
| Approved By | Chief Scientific Officer, CerviGen Therapeutics | _________________________ |